GUNNY BAG WEIGHING SYSTEM

Project Documentation

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

A Project Report

Submitted in partial fulfillment of the requirements for the

development and deployment of the

GUNNY BAG WEIGHING SYSTEM

at

UNITED RUBBER INDUSTRIES — UNIT 1

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Developed by

Kalpesh D. Dukhande

Under the esteemed guidance of

Sir Atharva Rasal

Atharva Automations

2026

# CERTIFICATE

This is to certify that the project entitled "Gunny Bag Weighing System" is the bonafide work of Kalpesh D. Dukhande, developed for United Rubber Industries — Unit 1 under Atharva Automations, submitted in partial fulfillment of the requirements for the deployment of an industrial weighing and label printing solution.

The system has been designed, developed, and documented under the guidance of Sir Atharva Rasal at Atharva Automations.

Date: September 2026

Guide: Sir Atharva Rasal

Developed By: Kalpesh D. Dukhande

________________________                                               	        ________________________

Sir Atharva Rasal                    				Kalpesh D. Dukhande

Guide, Atharva Automations            				           Developer

# ABSTRACT

The Gunny Bag Weighing System is a web-based industrial application developed for United Rubber Industries (Unit 1) that manages the complete workflow of weighing rubber compound bags. The system addresses the need for accurate, automated, and traceable bag weighing on the production floor by integrating a Modbus RTU weighing scale, a TSC thermal label printer, and a PostgreSQL database into a single unified platform.

The application maintains a master parts list with allowed weight tolerances per part, reads live weight from a Modbus RTU scale over a serial port, automatically validates each bag weigh-in against minimum and maximum thresholds, and prints thermal labels on each accepted weigh-in. It also tracks full weighing history with filtering, CSV export, and PDF download capabilities, and provides a real-time dashboard with statistics and recent activity.

The system is built using Next.js 16 with the App Router, Drizzle ORM for type-safe database operations, PostgreSQL as the primary database, and modbus-serial for scale communication. The TSC thermal printer is addressed directly via raw TCP sockets using TSPL commands, eliminating the need for an OS print driver.

The Gunny Bag Weighing System demonstrates the practical application of industrial automation, IoT device integration, and modern web technologies to solve real-world manufacturing challenges.

Keywords: Gunny Bag Weighing  •  Modbus RTU  •  TSC Thermal Printer  •  Next.js  •  PostgreSQL  •  Industrial IoT  •  TSPL

# ACKNOWLEDGEMENT

I would like to express my sincere gratitude and appreciation to all those who have supported and guided me throughout the development of the Gunny Bag Weighing System project.

First and foremost, I extend my heartfelt thanks to Sir Atharva Rasal for his invaluable guidance, constant encouragement, and insightful feedback that helped shape this project from conception to completion. His expertise and patience were instrumental in overcoming numerous technical challenges.

I am deeply grateful to United Rubber Industries (Unit 1) for providing the opportunity to develop and deploy this system in a real production environment, and for their cooperation during testing and commissioning.

My sincere thanks to the open-source community behind Next.js, Drizzle ORM, modbus-serial, pdfkit, and qrcode, whose libraries and frameworks provided the foundation upon which this application was built.

I would also like to acknowledge the TSC and Modbus hardware documentation that informed the printer and scale integration logic.

A special note of appreciation to my family and friends for their unwavering support, understanding, and encouragement during the intensive development phases of this project.

Lastly, I thank all the operators and staff at United Rubber Industries who provided valuable feedback that helped refine the user experience and functionality of the weighing system.

This project would not have been possible without the collective support of all these individuals and organizations.

________________________

Kalpesh D. Dukhande

Developer

# DECLARATION

I hereby declare that the project entitled "Gunny Bag Weighing System" developed for United Rubber Industries — Unit 1 under Atharva Automations has not been in any case duplicated or submitted to any other organization for the award of any degree or contract.

To the best of my knowledge, other than me, no one has submitted this project to any other organization.

The project is developed and documented as a complete industrial weighing and label printing solution.

________________________

Kalpesh D. Dukhande

Developer

# TABLE OF CONTENTS

Certificate

Abstract

Acknowledgement

Declaration

1    Introduction

1.1    Background

1.2    Objectives

1.3    Purpose, Scope, and Applicability

1.4    Achievements

1.5    Organization of Report

2    System Analysis

2.1    Methodology (Software Process Model)

2.2    Tools and Techniques

3    Requirements and Analysis

3.1    Problem Definition

3.2    Requirement Specification

3.3    Planning and Scheduling

3.4    Software and Hardware Requirements

3.5    Preliminary Product Description

3.6    Conceptual Model

3.7    Benefit Analysis

4    System Design

4.1    Architecture Overview

4.2    Use Case Diagram

4.3    Activity Diagram

4.4    Class Diagram

4.5    ER Diagram

4.6    Component Diagram

4.7    Deployment Diagram

4.8    Data Flow Diagram

4.9    Website Workflow Diagram

4.10    List of Tables

4.11    Screen Descriptions

5    Implementation and Testing

5.1    Implementation Approaches

5.2    Coding Details and Code Efficiency

5.3    Testing Approach

5.4    Modifications and Improvements

5.5    Test Cases

6    Results and Discussion

6.1    Test Reports

6.2    User Documentation

7    Conclusions

7.1    Conclusion

7.2    Limitations of the System

7.3    Future Scope of the Project

References

# CHAPTER 1: INTRODUCTION

The Gunny Bag Weighing System is a web-based application developed for United Rubber Industries (Unit 1) that manages the complete workflow of weighing rubber compound bags. In modern manufacturing environments, accurate weight measurement and traceability are critical for quality control, inventory management, and regulatory compliance.

The system integrates a Modbus RTU weighing scale over a serial port, a TSC thermal label printer over Ethernet, and a PostgreSQL database into a single unified platform. It maintains a master parts list with allowed weight tolerances per part, reads live weight from the scale, automatically records and validates each bag weigh-in against min/max thresholds, and prints thermal labels (TSC, 100×50mm) on each accepted weigh-in.

## 1.1  Background

The manufacturing sector worldwide is experiencing a significant digital transformation. In India, rubber and polymer processing industries face challenges related to manual weighing errors, inconsistent bag weights, lack of traceability, and time-consuming label printing processes.

Traditional weighing systems require operators to manually read the scale, calculate tolerances, and write labels by hand. The Gunny Bag Weighing System addresses these challenges by providing:

- Automated weight validation against part-specific tolerances

- Real-time label printing on accepted weigh-ins

- Complete digital record-keeping of all weighing operations

- Real-time dashboard for production monitoring

## 1.2  Objectives

The Gunny Bag Weighing System has been designed with comprehensive and measurable objectives:

### 1.2.1  Primary Technical Objectives

- Design and implement a robust Next.js 16 web application with 35+ API endpoints

- Integrate Modbus RTU weighing scale over serial port with real-time polling

- Implement weight stability detection using full min/max range tracking

- Create thermal label printing via raw TCP socket (TSPL commands)

- Implement QR code generation for machine-scannable labels

- Design PostgreSQL database schema with auto-migration on startup

### 1.2.2  User Experience Objectives

- Create a clean, professional interface with consistent navigation across all pages

- Implement responsive design for desktop and wide monitors

- Provide color-coded status indicators (OK/UNDERWEIGHT/OVERWEIGHT)

### 1.2.3  Industrial Operations Objectives

- Automate bag weigh-in recording and label printing

- Reduce manual weighing errors through real-time validation

- Enable batch label printing for multiple parts

- Support printer calibration (gap sensor re-teach)

### 1.2.4  Technical Excellence Objectives

- Scalable architecture with singleton scale service surviving HMR

- Auto-migration of database schema on startup

- Non-overlapping scale polling to prevent read queue build-up

- Denormalized history to preserve records even if parts are renamed/deleted

### 1.2.5  Innovation and Research Objectives

- Demonstrate practical Industrial IoT integration with web applications

- Create real-time stability detection algorithm preventing drift-triggered false positives

- Implement responsible fallback mechanisms (mock mode) for development

## 1.3  Purpose, Scope, and Applicability

### 1.3.1  Purpose

The purpose of the Gunny Bag Weighing System is to address the critical gap in accurate, traceable, and automated bag weighing in rubber compound manufacturing. The system aims to provide immediate, reliable weight validation and label printing while maintaining complete digital records for quality control and audit purposes.

### 1.3.2  Scope

Included Functionalities:

- Master parts list management with weight tolerances

- Live weight reading from Modbus RTU scale

- Automatic status determination (OK/UNDERWEIGHT/OVERWEIGHT)

- Auto-save and auto-print on stable weight (5 seconds)

- Thermal label printing (TSC, 100×50mm)

- Batch label printing for multiple parts

- Weighing history with filtering, CSV export, PDF download

- Dashboard statistics and recent activity

- Printer calibration (gap sensor re-teach)

- Mock scale mode for development/testing

Assumptions:

- Operators have basic computer literacy

- The scale and printer are properly connected and configured

- Network connectivity exists between server and printer

Limitations:

- Windows-only for scale communication (COM ports)

- History API returns max 500 records per query

- QR code part number must be < 20 characters

### 1.3.3  Applicability

Direct Applications:

- Rubber compound bag weighing and validation

- Production floor weight recording and thermal label printing

- Quality control and traceability

Indirect Applications:

- Any manufacturing process requiring weight validation

- Industrial IoT integration reference and automated label printing systems

## 1.4  Achievements

Technical Competencies Gained:

- Mastery of Next.js 16 App Router and API routes

- Expertise in Modbus RTU communication over serial ports

- Advanced thermal printer integration using TSPL commands

- Database design with Drizzle ORM and PostgreSQL

- Implementation of real-time weight stability algorithms

Domain Knowledge Acquired:

- Understanding of industrial weighing systems and Modbus RTU protocol

- Awareness of thermal printer command languages (TSPL)

- Understanding of weight tolerance and validation principles

## 1.5  Organization of Report

This project report has been systematically structured into seven comprehensive chapters documenting the entire software development lifecycle.

- Chapter 1: Introduction — Context, background, and project objectives

- Chapter 2: System Analysis — Methodology and tools used

- Chapter 3: Requirements and Analysis — Problem definition and requirements

- Chapter 4: System Design — Architecture, diagrams, and database design

- Chapter 5: Implementation and Testing — Code details and test cases

- Chapter 6: Results and Discussion — Test reports and user documentation

- Chapter 7: Conclusions — Achievements, limitations, and future scope

# CHAPTER 2: SYSTEM ANALYSIS

## 2.1  Methodology (Software Process Model)

The Gunny Bag Weighing System was developed using an Iterative Model as the primary software process model, incorporating Agile principles for continuous improvement.

### 2.1.1  Iterative Model Framework

Iteration 1: Foundation and Hardware Integration

- Objectives: Establish technical feasibility of scale and printer communication

- Deliverables: Basic Next.js framework, Modbus scale prototype, printer test

- Decisions: Proceed with modbus-serial, raw TCP for printer

Iteration 2: Core Module Development

- Objectives: Develop master parts list and weighing history modules

- Deliverables: Working parts CRUD, history recording, status computation

- Decisions: Implement Drizzle ORM, denormalized history

Iteration 3: Live Weighing and Auto-Print

- Objectives: Integrate live weighing page with auto-save and auto-print

- Deliverables: Functional live weighing, stability algorithm, auto-print cycle

- Decisions: Implement full min/max spread tracking, rearm on weight drop

Iteration 4: Dashboard, History, and Optimization

- Objectives: Complete dashboard, history filters, CSV/PDF export

- Deliverables: Dashboard statistics, history filtering, export functionality

- Decisions: Limit API to 500 records, client-side pagination

### 2.1.2  Risk-Driven Approach

Technical Risks:

- Modbus Communication Complexity: Addressed through early prototyping

- Printer TCP Connectivity: Resolved through raw socket implementation with 5s timeout

- Weight Stability False Positives: Mitigated through full min/max spread tracking

Project Risks:

- Hardware Availability: Managed through mock mode fallback

- Scope Creep: Controlled through iteration boundary definitions

### 2.1.3  Agile Integration

- 2-week sprints within each iteration with daily progress tracking

- User story mapping, backlog management, and sprint reviews

- Test-driven development (TDD) for critical algorithms

### 2.1.4  Quality Assurance

- Code review and integration testing with hardware for each iteration

- Performance validation for scale polling and print quality verification

## 2.2  Tools and Techniques

The following technologies were used in the development of the Gunny Bag Weighing System:

| Category | Technology | Version | Purpose |
| --- | --- | --- | --- |
| Frontend Framework | Next.js | 16.2.6 | Full-stack React framework (App Router) |
| UI Library | React | 19.2.6 | UI library |
| Styling | Tailwind CSS | 4.1.17 | Utility-first styling |
| Icons | Lucide React | 1.39.0 | Icon library |
| QR Code (Client) | qrcode.react | 4.2.0 | QR code rendering in browser |
| Date Utilities | date-fns | 4.4.0 | Date formatting utilities |
| Backend API | Next.js API Routes | 16.2.6 | Server-side REST API (Node.js runtime) |
| ORM | Drizzle ORM | 0.45.2 | Type-safe database ORM |
| Migrations | drizzle-kit | 0.31.10 | Schema migrations |
| Database Driver | pg | 8.20.0 | PostgreSQL Node.js driver |
| Scale Communication | modbus-serial | 8.0.25 | Modbus RTU over serial port |
| PDF Generation | pdfkit | 0.20.2 | Server-side PDF generation |
| QR Code (Server) | qrcode | 1.5.4 | Server-side QR code matrix generation |
| Database | PostgreSQL | 14+ | Primary relational database |
| Runtime | Node.js | 20+ | JavaScript runtime |
| Language | TypeScript | 5.9.3 | Static typing |
| Linting | ESLint | 9.39.4 | Code linting |

# CHAPTER 3: REQUIREMENTS AND ANALYSIS

## 3.1  Problem Definition

The Gunny Bag Weighing System addresses critical gaps in manual weighing and traceability in rubber compound manufacturing.

### 3.1.1  Current System Analysis

- Manual weight reading requires operators to visually read the scale and manually calculate tolerances

- Human error leads to inconsistent bag weights and incorrect status classification

- No real-time validation means underweight/overweight bags are not detected immediately

- Manual label writing is time-consuming and prone to transcription errors

- No digital record-keeping makes traceability and audit difficult

### 3.1.2  Feasibility Study

Technical Feasibility:

The project is technically feasible as modbus-serial is available for Modbus RTU communication, Next.js supports full-stack development with API routes, and raw TCP sockets enable TSPL printer communication.

Economic Feasibility:

Highly economical with development tools costing $0 using open-source technologies. The TSC printer and Modbus scale are existing hardware assets.

Return on Investment:

- Elimination of manual weighing errors

- Complete traceability for quality audits

- Real-time production monitoring

- Reduced material waste from incorrect weights

## 3.2  Requirement Specification

### 3.2.1  Functional Requirements

| # | Requirement |
| --- | --- |
| FR-01 | Maintain a master list of rubber compound parts with part number, description, weight tolerances, and bag quantity |
| FR-02 | Add, edit, and delete parts from the master list |
| FR-03 | Display a read-only inventory view of all parts |
| FR-04 | Connect to a Modbus RTU weighing scale over a serial port |
| FR-05 | Display live weight readings updated every second |
| FR-06 | Determine bag status: OK, UNDERWEIGHT, or OVERWEIGHT based on bag-level min/max thresholds |
| FR-07 | Auto-save a weighing record and print a label when the scale is stable for 5 seconds |
| FR-08 | Allow manual recording of a weigh-in with operator name and remarks |
| FR-09 | Print a thermal label (TSC, 100×50mm) with part details, QR code, and measured weight |
| FR-10 | Support batch label printing for multiple parts at once |
| FR-11 | Maintain a searchable, filterable weighing history (up to 500 records per query) |
| FR-12 | Export weighing history as CSV |
| FR-13 | Generate and download PDF labels and weighing record documents |
| FR-14 | Show dashboard statistics: total parts, total weighings, status breakdown, today's count |
| FR-15 | Support printer calibration (gap sensor re-teach) |
| FR-16 | Fall back to mock scale readings when the physical scale is offline (dev/test mode) |

### 3.2.2  Non-Functional Requirements

| # | Requirement |
| --- | --- |
| NFR-01 | Auto-migrate database schema on startup — no manual migration commands needed |
| NFR-02 | All timestamps stored in UTC and displayed in IST (Asia/Kolkata) |
| NFR-03 | Printer communication via raw TCP socket — no print driver required |
| NFR-04 | Scale polling must be non-overlapping to prevent read queue build-up |
| NFR-05 | Weight stability detection must track full min/max range, not just consecutive differences, to prevent drift-triggered false positives |
| NFR-06 | Label QR codes must be machine-scannable (ECC M, Model 2) |
| NFR-07 | UI must be responsive and usable on standard desktop / wide monitors |

## 3.3  Planning and Scheduling

### 3.3.1  Project Schedule

| Phase | Start Date | End Date | Duration |
| --- | --- | --- | --- |
| Market Research and Requirement Gathering | 15-Jul-25 | 21-Jul-25 | 7 days |
| Technical Specification and Architecture Design | 22-Jul-25 | 28-Jul-25 | 7 days |
| Basic Framework and Hardware Integration | 29-Jul-25 | 05-Aug-25 | 8 days |
| Scale Communication Module Implementation | 06-Aug-25 | 15-Aug-25 | 10 days |
| Printer Integration and Label Design | 16-Aug-25 | 26-Aug-25 | 11 days |
| Database Design and Implementation | 27-Aug-25 | 09-Sep-25 | 14 days |
| Live Weighing Module Development | 10-Sep-25 | 20-Sep-25 | 11 days |
| Weighing History and Filtering | 21-Sep-25 | 05-Oct-25 | 15 days |
| Dashboard and Statistics Development | 06-Oct-25 | 20-Oct-25 | 15 days |
| Frontend Integration and UI Enhancement | 21-Oct-25 | 05-Nov-25 | 16 days |
| Unit Testing and Module Validation | 06-Nov-25 | 20-Nov-25 | 15 days |
| Integration Testing and Bug Fixing | 21-Nov-25 | 10-Dec-25 | 20 days |
| System Testing and Performance Optimization | 11-Dec-25 | 05-Jan-26 | 26 days |
| User Acceptance Testing | 06-Jan-26 | 20-Jan-26 | 15 days |
| Security Testing and Vulnerability Assessment | 21-Jan-26 | 05-Feb-26 | 16 days |
| Documentation and Report Writing | 06-Feb-26 | 20-Feb-26 | 15 days |
| Final Review and Revisions | 21-Feb-26 | 27-Feb-26 | 7 days |
| Final Deployment and Submission | 28-Feb-26 | 03-Mar-26 | 4 days |

## 3.4  Software and Hardware Requirements

### 3.4.1  Software Requirements

Development Environment:

- Operating System: Windows 10/11  |  Node.js 20+  |  PostgreSQL 14+

- Next.js 16.2.6  |  Drizzle ORM 0.45.2  |  modbus-serial 8.0.25

- Git Version Control System  |  Visual Studio Code

Frontend Technologies:

- HTML5, CSS3, JavaScript ES6+  |  Tailwind CSS 4.1.17  |  Lucide React

- qrcode.react 4.2.0  |  date-fns 4.4.0  |  Chrome 90+, Firefox 85+

### 3.4.2  Hardware Requirements

- Development Workstation: Intel i5+, 8GB RAM, 1920×1080, USB port for RS-485

- Weighing Scale: Modbus RTU output, RS-485 or RS-232 interface

- USB-RS485 Adapter: For scale connection

- TSC Thermal Printer: 203 dpi, Ethernet interface, 100×50mm labels

## 3.5  Preliminary Product Description

| Module | Key Features |
| --- | --- |
| Dashboard | Total parts, total weighings, today's count, status breakdown, recent activity |
| Master List | Search, create, edit, delete parts, batch print, PDF download, calibrate printer |
| Inventory | Read-only view, client-side filtering, summary cards |
| Live Weighing | Scale status bar, part selection, weight display, stability indicator, auto-print |
| Weighing History | Filters, pagination, CSV export, PDF download, delete record |
| Label Printing | Thermal label (100×50mm), QR code, batch printing, PDF label, calibration |

## 3.6  Conceptual Model (System Flow Chart)

Browser (Next.js Client)

Dashboard | Master List | Inventory | Weighing | History

|  HTTP / fetch

Next.js App Server

API Routes: /api/parts  /api/history  /api/scale  /api/stats  /api/print

Background Services: ScaleService (singleton, globalThis)

|

+─────────────────────────────────+

|                                 |

[PostgreSQL DB]              [Hardware Devices]

- parts                      - Scale: Modbus RTU, COM7, 9600 baud

- weighing_history            - Printer: TSC, 192.168.1.75:9100

## 3.7  Cost Benefit Analysis

### 3.7.1  Benefit Analysis

- Elimination of manual weighing errors through automated validation

- Reduced label printing time — thermal labels print in seconds

- Complete traceability — every weigh-in recorded with timestamp, operator, and status

- Real-time production monitoring through the dashboard

- Reduced material waste from immediate detection of incorrect weights

# CHAPTER 4: SYSTEM DESIGN

## 4.1  Architecture Overview

The Gunny Bag Weighing System follows a three-tier architecture: Presentation Layer (Next.js client), Application Layer (Next.js API routes and background services), and Data Layer (PostgreSQL database).

- Single server process: Scale service runs as a singleton on globalThis to survive Next.js HMR

- Auto-migration: instrumentation.ts triggers database migrations on startup

- No print driver: TSC printer addressed via raw TCP (port 9100) using TSPL commands

- Denormalized history: weighing_history stores part_number and description alongside part_id

## 4.2  Use Case Diagram

GUNNY BAG WEIGHING SYSTEM

[Dashboard]         [Master List]        [Inventory]

View Stats           Create Part          View Parts

Recent Activity      Edit Part            Filter

Delete Part

[Live Weighing]     [History]            [Printing]

Read Weight          Search               Thermal Label

Select Part          Filter               PDF Label

Auto-Print           Export CSV           Calibrate Printer

Manual Record        PDF Download

<<OPERATOR>>

## 4.3  Activity Diagram

[Start] → Access Website → Dashboard

|

+──────────────── [Main Menu] ──────────────────────+

|                      |                            |

[Live Weighing]       [Master List]                 [History]

Select Part            Create/Edit Part             Filter

Read Weight            Delete Part                  Export CSV

Auto-Print             Batch Print                  PDF Download

|                      |                            |

+──────────────────────+────────────────────────────+

[Return to Dashboard]

## 4.4  Class Diagram

┌──── Part ─────────────────────────────────────────────────┐

│ - id, part_number, description                           │

│ - min_weight, max_weight, quantity, actual_weight        │

│ + create(), update(), delete(), getBagMin/MaxWeight()    │

└────────────────────────┬──────────────────────────────────┘

| 1:*

┌──── WeighingHistory ───────────────────────────────────────┐

│ - id, part_id (FK), part_number, description             │

│ - actual_weight, quantity, status, operator_name, remarks │

│ + create(), getHistory(), delete(), exportCSV()          │

└───────────────────────────────────────────────────────────┘

┌──── ScaleService ─────────────────────────────────────────┐

│ - connected, port, baudRate, slaveId, divisor             │

│ + connect(), readOnce(), getCurrentReading()              │

└───────────────────────────────────────────────────────────┘

┌──── PrinterClient ────────────────────────────────────────┐

│ - ip, port                                                │

│ + send(tspl), generateGunnyBagTSPL(data), calibrate()    │

└───────────────────────────────────────────────────────────┘

## 4.5  ER Diagram

┌──────────────── parts ─────────────────────────────────────┐

│ PK  id             SERIAL                                  │

│     part_number    VARCHAR(100) UNIQUE NOT NULL            │

│     description    VARCHAR(500) NOT NULL                   │

│     min_weight     NUMERIC(10,3) NOT NULL                  │

│     max_weight     NUMERIC(10,3) NOT NULL                  │

│     quantity       INTEGER NOT NULL                        │

│     actual_weight  NUMERIC(10,3) DEFAULT 0                 │

└───────────────────────┬────────────────────────────────────┘

| 1:*

┌──────────── weighing_history ──────────────────────────────┐

│ PK  id             SERIAL                                  │

│ FK  part_id        INTEGER → parts(id) ON DELETE CASCADE   │

│     part_number    VARCHAR(100)  [denormalized]            │

│     actual_weight  NUMERIC(10,3) NOT NULL                  │

│     status         VARCHAR(20)   OK/UNDERWEIGHT/OVERWEIGHT │

│     operator_name  VARCHAR(200)                            │

│     recorded_at    TIMESTAMPTZ DEFAULT NOW()               │

└────────────────────────────────────────────────────────────┘

## 4.6  Component Diagram

┌─ Client Layer ──────────────────────────────────────────────┐

│  Dashboard  MasterList  Inventory  Weighing  History        │

└────────────────────── HTTP/HTTPS ───────────────────────────┘

┌─ Application Layer ─────────────────────────────────────────┐

│  Next.js API Routes                                         │

│  /api/parts  /api/history  /api/scale  /api/stats  /api/print│

│  Background: ScaleService (singleton, globalThis)           │

└────────────┬────────────────────────────────────────────────┘

|

┌─────────┴──────────┐

|                    |

[PostgreSQL DB]    [Hardware Layer]

- parts            - Scale: Modbus RTU, COM7, 9600

- weighing_history  - Printer: TSC Ethernet, :9100

## 4.7  Deployment Diagram

┌─── Factory Floor ────────────────────────────────────────────┐

│  ┌─ Windows Machine (Server) ───────────────────────────────┐│

│  │  Next.js: API Routes | ScaleService | PrinterClient      ││

│  └──────────────────────────────────────────────────────────┘│

│         │ USB           │ Ethernet          │ HTTP            │

│  [Weighing Scale]  [TSC Printer]     [Operator Browser]      │

│  Modbus RTU        192.168.1.75:9100                         │

│  ┌─ PostgreSQL Database ─────────────────────────────────┐   │

│  │  parts  |  weighing_history                           │   │

│  └──────────────────────────────────────────────────────┘   │

└──────────────────────────────────────────────────────────────┘

## 4.8  Data Flow Diagram

Level 0 – Context:

[Operator] ──→ [Gunny Bag Weighing System] ──→ [TSC Printer]

[Scale (Modbus)] ──→ [System] ──→ [PostgreSQL DB]

Level 1 – Major Processes:

[Operator] → [Part Management] → [Parts DB]

[Scale]    → [Weight Reading]  → [Live Weight Display]

[Operator] → [Weighing Record] → [History DB] → [Label Printing] → [Printer]

Level 2 – Weighing Detail:

[Read Weight] → [Check Status] → [Display Status]

[Stable for 5s?] → Yes → [Save Record + Print Label]

→ No  → [Continue Reading]

## 4.9  Website Workflow Diagram

[Dashboard] → [Master List] → [Inventory]

|              |               |

▼              ▼               ▼

[Live Weighing] → [History] → [Label Printing]

Live Weighing: Select Part → Read Weight → Auto-Print → Save Record

Master List:   Create/Edit/Delete → Batch Print → PDF Label

History:       Filter → Export CSV → PDF Download → Delete

## 4.10  List of Tables

### Table 4.10.1: parts — Master Parts List

| Field Name | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | SERIAL | PRIMARY KEY | Unique part identifier |
| part_number | VARCHAR(100) | NOT NULL, UNIQUE | Part number (stored uppercase) |
| description | VARCHAR(500) | NOT NULL | Part description |
| min_weight | NUMERIC(10,3) | NOT NULL | Per-item minimum weight (kg) |
| max_weight | NUMERIC(10,3) | NOT NULL | Per-item maximum weight (kg) |
| quantity | INTEGER | NOT NULL | Pieces per bag |
| actual_weight | NUMERIC(10,3) | DEFAULT 0 | Last measured weight |
| created_at | TIMESTAMPTZ | DEFAULT NOW() | Record creation timestamp |
| updated_at | TIMESTAMPTZ | DEFAULT NOW() | Last update timestamp |

### Table 4.10.2: weighing_history — All Weighing Records

| Field Name | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | SERIAL | PRIMARY KEY | Unique record identifier |
| part_id | INTEGER | FK → parts(id) CASCADE | References the part |
| part_number | VARCHAR(100) | NOT NULL | Denormalized part number |
| description | VARCHAR(500) | NOT NULL | Denormalized description |
| actual_weight | NUMERIC(10,3) | NOT NULL | Measured bag weight (kg) |
| quantity | INTEGER | NOT NULL | Pieces in this bag |
| status | VARCHAR(20) | NOT NULL | OK / UNDERWEIGHT / OVERWEIGHT |
| operator_name | VARCHAR(200) |  | Operator name |
| remarks | TEXT |  | Additional remarks |
| recorded_at | TIMESTAMPTZ | DEFAULT NOW() | Record timestamp |

### Status Computation:

bag_min = min_weight × quantity

bag_max = max_weight × quantity

actual_weight < bag_min  →  UNDERWEIGHT

actual_weight > bag_max  →  OVERWEIGHT

else                     →  OK

## 4.11  Screen Descriptions

### 4.11.1  Dashboard Page  (Route: /)

The dashboard is the home screen providing a quick overview of system activity.

- 6 Stat Cards: Total Parts, Total Weighings, Today's Weighings, OK / Overweight / Underweight counts

- Quick Action Cards: Links to Live Weighing, Master List, and History

- Recent Activity Table: Last 5 weighing records with status badges

### 4.11.2  Master List Page  (Route: /master)

| Feature | Description |
| --- | --- |
| Search | Debounced 350ms text search across part number and description |
| Create/Edit Part | Modal form with live bag-weight preview as you type |
| Delete Part | Confirmation modal warning that history records will also be deleted |
| Batch Select/Print | Checkbox per row, prints one label per selected part (400ms delay) |
| Print Label | Single-part label print from Actions column |
| Download PDF | A6 PDF label download |
| Calibrate Printer | Sends TSPL calibration sequence |

### 4.11.3  Inventory Page  (Route: /inventory)

Read-only view of all parts with client-side filtering by text, quantity range, and weight range.

### 4.11.4  Live Weighing Page  (Route: /weighing)

- Scale Status Bar: Live connection state with Pause/Resume button

- Part Selection: Search modal to find and select a part. Pre-select via URL: /weighing?part=PN-0001

- Live Weight Display: Large monospace font, updates every 500ms, 3 decimal places

- Stability: Tracks min/max spread over rolling 5-second window. Stable when spread ≤ 0.005 kg for 5s

- Auto-Print: Stable weight + real scale → auto-save + auto-print + reset

### 4.11.5  Weighing History Page  (Route: /history)

| Feature | Description |
| --- | --- |
| Part number filter | Case-insensitive text search (ILIKE) |
| Status filter | All / OK / OVERWEIGHT / UNDERWEIGHT |
| Date range filter | From date / To date pickers |
| Pagination | 15 records per page with Previous/Next navigation |
| Export CSV | Downloads all matching records as CSV (max 500) |
| PDF download | Downloads full A4 record PDF |
| Delete record | Confirmation modal before deletion |

### 4.11.6  Label Printing

- Thermal label: 100mm × 50mm at 203 dpi (800 × 400 dots)

- Layout includes: Company name, Part No., Qty/Bag, Min–Max weights, Expected Bag weight, Date, Actual weight, Status, QR code

- QR code: Encodes part number. ECC M, Model 2

- Communication: Raw TCP socket to PRINTER_IP:9100. 5-second timeout

- PDF Label: A6 landscape PDF via PDFKit for digital download/archiving

# CHAPTER 5: IMPLEMENTATION AND TESTING

## 5.1  Implementation Approaches

The system was developed using a modular approach in the following sequence:

- 1. Database layer and connection utilities

- 2. Scale service with Modbus RTU communication

- 3. Parts management (CRUD operations)

- 4. Live weighing page with stability detection

- 5. Auto-save and auto-print workflow

- 6. Weighing history with filtering and export

- 7. Dashboard with statistics

- 8. Label printing (thermal and PDF)

- 9. Printer calibration

## 5.2  Coding Details and Code Efficiency

### 5.2.1  Scale Service Connection

class ScaleService {

async connect() {

this.client = new ModbusRTU();

await this.client.connectRTUBuffered(port, { baudRate });

this.client.setID(slaveId);

this.startPolling();

}

async readOnce() {

const data = await this.client.readInputRegisters(register, quantity);

return this.decodeRegister(data.data[0]);

}

}

### 5.2.2  Weight Stability Algorithm

class WeightStability {

private readings: { weight: number; time: number }[] = [];

private readonly STABLE_DURATION_MS = 5000;

private readonly WEIGHT_TOLERANCE_KG = 0.005;

update(weight: number): { isStable: boolean } {

const now = Date.now();

this.readings.push({ weight, time: now });

this.readings = this.readings.filter(

r => now - r.time <= this.STABLE_DURATION_MS

);

const weights = this.readings.map(r => r.weight);

const spread = Math.max(...weights) - Math.min(...weights);

const coversDuration = now - this.readings[0].time >= this.STABLE_DURATION_MS;

return { isStable: spread <= this.WEIGHT_TOLERANCE_KG && coversDuration };

}

}

### 5.2.3  TSPL Label Generation

function generateGunnyBagTSPL(data: LabelData): string {

return `

SIZE 100 mm, 50 mm

GAP 2 mm, 0 mm

CLS

TEXT 280, 30, '2', 0, 1, 1, 'UNITED RUBBER INDUSTRIES'

BARCODE 30, 100, 'QRCODE', 80, 1, 0, 1, 1, '${data.partNumber}'

TEXT 280, 80, '2', 0, 1, 1, 'Part No.: ${data.partNumber}'

TEXT 280, 120, '2', 0, 1, 1, 'Qty/Bag: ${data.quantity} pcs'

PRINT 1, 1

`;

}

### 5.2.4  Code Efficiency

- Database queries use parameterized statements via Drizzle ORM

- Connection pooling is implemented to reuse database connections

- Singleton pattern for scale service survives Next.js HMR

- Non-overlapping polling prevents read queue build-up

- Debounced search (350ms) reduces API calls

- Batch printing with 400ms delay prevents printer buffer overrun

## 5.3  Testing Approach

### 5.3.1  Unit Testing

| Module Tested | Test Cases | Expected Outcome |
| --- | --- | --- |
| Part Creation | Valid data, duplicate, invalid weights | Success for valid, error for duplicates |
| Status Computation | OK, UNDERWEIGHT, OVERWEIGHT scenarios | Correct status returned |
| Weight Stability | Stable, drifting, stale data | Stable only when criteria met |
| Modbus Decode | u16, s16, u32be, u32le, s32be, s32le | Correct weight value |
| TSPL Generation | Valid data, long part number | Valid TSPL or error |
| CSV Export | Multiple records, empty records | Correct CSV format |

### 5.3.2  Integration Testing

- Scale Service + API: Live weight reading through /api/scale

- Part + History: Creating a weighing record for a selected part

- Auto-Print Workflow: Stability detection → record save → label print

- Database + Auto-Migration: Startup migration process verified

## 5.4  Modifications and Improvements

- Stability algorithm improved to track full min/max spread instead of consecutive differences

- Auto-print rearm logic added requiring weight drop to ≤ 0.005 kg before next cycle

- Error logging throttled to once every 30 seconds for scale errors

- Batch print delay increased to 400ms to prevent printer buffer overrun

- Mock mode protection added to prevent phantom records during development

- History denormalization implemented to preserve records after part deletion

## 5.5  Test Cases

### Table 5.1: Part Management Test Cases

| ID | Test Case | Input | Expected Result | Result |
| --- | --- | --- | --- | --- |
| TC01 | Valid Part Creation | partNumber="PN-001", minWeight=0.5, maxWeight=1.0, quantity=100 | Part created successfully | Pass |
| TC02 | Duplicate Part Number | Same part number as TC01 | Error "Part number already exists" | Pass |
| TC03 | Invalid Weight Range | minWeight=1.0, maxWeight=0.5 | Error "Min weight must be less than max" | Pass |
| TC04 | Zero Quantity | quantity=0 | Error "Quantity must be at least 1" | Pass |
| TC05 | Edit Part | Update description | Part updated successfully | Pass |
| TC06 | Delete Part | Part ID exists | Part deleted, history cascaded | Pass |

### Table 5.2: Weighing Test Cases

| ID | Test Case | Input | Expected Result | Result |
| --- | --- | --- | --- | --- |
| TC07 | OK Status | Weight within min-max | Status = OK | Pass |
| TC08 | Underweight Status | Weight below min | Status = UNDERWEIGHT | Pass |
| TC09 | Overweight Status | Weight above max | Status = OVERWEIGHT | Pass |
| TC10 | Live Weight Reading | Scale connected | Weight displayed in kg | Pass |
| TC11 | Stable Weight Detection | Weight constant for 5s | Stability = true | Pass |
| TC12 | Drifting Weight | Weight slowly changing | Stability = false | Pass |
| TC13 | Auto-Print | Stable weight + part selected | Record saved, label printed | Pass |
| TC14 | Auto-Print Rearm | Weight drops to 0 | Auto-print rearmed | Pass |
| TC15 | Manual Record | Record button clicked | Record saved without print | Pass |

### Table 5.3: Printer Test Cases

| ID | Test Case | Input | Expected Result | Result |
| --- | --- | --- | --- | --- |
| TC16 | Single Label Print | Valid part ID | Label printed | Pass |
| TC17 | Batch Label Print | Multiple part IDs | Labels printed with delay | Pass |
| TC18 | PDF Label Download | Valid part ID | PDF downloaded | Pass |
| TC19 | Printer Calibration | Calibrate button | Calibration sequence sent | Pass |
| TC20 | Printer Unreachable | Wrong IP | Error "Printer unreachable" | Pass |

### Table 5.4: History Test Cases

| ID | Test Case | Input | Expected Result | Result |
| --- | --- | --- | --- | --- |
| TC21 | Part Number Filter | Part number search | Matching records displayed | Pass |
| TC22 | Status Filter | Status = OK | Only OK records displayed | Pass |
| TC23 | Date Range Filter | From and To dates | Records in range displayed | Pass |
| TC24 | CSV Export | Filter applied | CSV file downloaded | Pass |
| TC25 | PDF Download | Record ID | A4 PDF downloaded | Pass |
| TC26 | Delete Record | Record ID | Record deleted | Pass |

### Table 5.5: Dashboard Test Cases

| ID | Test Case | Input | Expected Result | Result |
| --- | --- | --- | --- | --- |
| TC27 | Total Parts | Parts exist | Correct count displayed | Pass |
| TC28 | Total Weighings | Records exist | Correct count displayed | Pass |
| TC29 | Today's Weighings | Records from today | Correct count displayed | Pass |
| TC30 | Status Breakdown | Mixed statuses | Correct counts displayed | Pass |
| TC31 | Recent Activity | Records exist | Last 5 records displayed | Pass |

# CHAPTER 6: RESULTS AND DISCUSSION

## 6.1  Test Reports

The Gunny Bag Weighing System underwent comprehensive testing across all functional modules. A total of 31 test cases were executed with a 100% success rate.

| Module | Test Cases | Passed | Failed | Success Rate |
| --- | --- | --- | --- | --- |
| Part Management | 6 | 6 | 0 | 100% |
| Weighing Module | 9 | 9 | 0 | 100% |
| Printer Module | 5 | 5 | 0 | 100% |
| History Module | 6 | 6 | 0 | 100% |
| Dashboard Module | 5 | 5 | 0 | 100% |
| TOTAL | 31 | 31 | 0 | 100% |

✓  Overall Result: 31/31 Test Cases Passed — 100% Success Rate

Performance testing revealed that API responses average under 2 seconds, scale polling executes every 500ms–1000ms, and PDF generation completes within 3–5 seconds. The system successfully handles multiple concurrent users.

All identified issues during testing were logged and resolved. The system has been successfully deployed and is operational at United Rubber Industries (Unit 1).

## 6.2  User Documentation

The Gunny Bag Weighing System is designed to be intuitive and user-friendly, requiring minimal training for operators.

### Getting Started

Access the application through any modern web browser at the designated URL. The system runs as a local server on a Windows machine connected to both the scale (USB/RS-485) and the label printer (Ethernet).

### Dashboard

Displays key statistics including total parts, total weighings, today's count, and status breakdown. Quick action cards provide access to Live Weighing, Master List, and History.

### Master List

Create, edit, and delete parts with part number, description, min/max weight, and quantity. Batch selection enables printing labels for multiple parts at once.

### Live Weighing

The core operational screen displays live weight from the scale. When the weight is stable for 5 seconds, the system auto-saves and auto-prints. Manual controls are available for recording without printing.

### Weighing History

Full audit trail with filtering by part number, status, and date range. Records can be exported as CSV or downloaded as PDF.

### Label Printing

Thermal labels (100×50mm) include part number, description, quantity, min-max weight, expected bag weight, date, actual weight, and QR code. Printer calibration can be performed from the Master List page.

# CHAPTER 7: CONCLUSIONS

## 7.1  Conclusion

The Gunny Bag Weighing System project has successfully achieved its objective of developing a web-based industrial weighing and label printing platform that provides accessible, accurate, and traceable bag weighing for United Rubber Industries (Unit 1).

Through extensive testing with 31 test cases achieving a 100% success rate, the system has demonstrated reliability, performance, and user-friendliness. The project successfully bridges the gap between manual weighing processes and automated industrial systems.

### Significance of the System:

- Elimination of Manual Weighing Errors — Automated validation ensures every bag is within tolerance

- Complete Traceability — Every weigh-in recorded with timestamp, operator, and status

- Real-Time Production Monitoring — Dashboard provides instant visibility

- Reduced Material Waste — Immediate detection of underweight/overweight bags

- Operator Efficiency — Auto-print reduces manual steps and label writing

- Scalability — System can handle multiple parts and high volumes

- Fallback Safety — Mock mode enables development without physical hardware

## 7.2  Limitations of the System

- Windows-only for scale: modbus-serial targets Windows COM ports. On Linux, change SCALE_PORT to /dev/ttyUSB0.

- History limit: /api/history returns at most 500 records per query. Use CSV export for full data.

- QR code length: Part number must be < 20 characters to fit QR matrix on 100×50mm label.

- Auto-print rearm: After auto-print, operator must remove bag to drop weight ≤ 0.005 kg to trigger next cycle.

- Printer TCP timeout: 5 seconds. Unreachable printer returns 500 error with 'Printer unreachable' message.

- All timestamps in UTC: Stored as TIMESTAMP WITH TIME ZONE. Displayed in IST (Asia/Kolkata, UTC+5:30).

- Label PDF vs thermal: PDF for digital download/archiving; thermal label sent to physical TSC printer.

## 7.3  Future Scope of the Project

The Gunny Bag Weighing System has significant potential for future enhancements:

- Multi-language support: Make the platform accessible to operators across different regions

- Mobile application: Native app for iOS and Android with push notifications

- ERP/MES Integration: Connect with existing enterprise systems for seamless data flow

- Advanced analytics: Predictive modeling for weight trends and quality issues

- Cloud deployment: Move from local server to cloud for remote access

- Barcode scanning: Part selection via barcode scanner for faster operation

- Offline mode: Allow basic functionality without network connectivity

- Automated reporting: Scheduled email reports for production managers

- Multiple scale support: Connect multiple scales for parallel weighing stations

- Role-based access control: Different permission levels for operators, supervisors, and admins

The future scope of the Gunny Bag Weighing System is vast, with opportunities to evolve from a local weighing solution into a comprehensive industrial automation platform.

# REFERENCES

[1]  Next.js Documentation. Next.js 16 — The React Framework for the Web.

Available at: https://nextjs.org/docs

[2]  Drizzle ORM Documentation. TypeScript ORM for SQL Databases.

Available at: https://orm.drizzle.team/

[3]  PostgreSQL Documentation. PostgreSQL 14 — Relational Database Management System.

Available at: https://www.postgresql.org/docs/

[4]  modbus-serial Documentation. Modbus RTU/TCP Client for Node.js.

Available at: https://github.com/yaacov/node-modbus-serial

[5]  TSC Printer Documentation. TSPL/TSPL2 Programming Guide.

Available at: https://www.tscprinters.com/

[6]  pdfkit Documentation. PDF Generation Library for Node.js.

Available at: https://pdfkit.org/

[7]  qrcode Documentation. QR Code Generation Library.

Available at: https://github.com/soldair/node-qrcode

[8]  Tailwind CSS Documentation. Utility-First CSS Framework.

Available at: https://tailwindcss.com/docs

[9]  Node.js Documentation. Node.js 20+ Runtime.

Available at: https://nodejs.org/docs/

[10]  TypeScript Documentation. TypeScript 5.9.

Available at: https://www.typescriptlang.org/docs/

[11]  Lucide React. Icon Library for React.

Available at: https://lucide.dev/

[12]  date-fns. Modern JavaScript Date Utility Library.

Available at: https://date-fns.org/

[13]  Modbus Organization. Modbus Application Protocol Specification V1.1b3.

Available at: https://modbus.org/

[14]  IEEE 802.3. Ethernet Standard.

Available at: https://standards.ieee.org/

[15]  ISO 9001:2015. Quality Management Systems — Requirements.

Available at: https://www.iso.org/

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Documentation prepared: September 2026  |  Atharva Automations

Developed by Kalpesh D. Dukhande under the guidance of Sir Atharva Rasal